TUTORIAL: External Interrupt

LED Toggle with Push-Button

Name:  An Gyeonheal				ID: 21900416

I. Introduction

In this tutorial, we will learn how to use External Interrupt. We will create functions that capture the falling edge trigger by pushing a button using an external interrupt.

The objectives of this lab are to learn how to

Configure External input (EXTI) interrupt with NVIC

Create your own functions for configuration of interrupts

Hardware

NUCLEO -F411RE

Software

Keil uVision IDE, CMSIS, EC_HAL

Documentation

STM32 Reference Manual

II. Basics of External Interrupt (EXTI)

A. Register List

List of external interrupt (EXTI) registers used in this tutorial [Reference Manual ch7, ch10.2]

| Type | Register Name | Description |
| --- | --- | --- |
| SYSCFG | SYSCFG_EXTICRx | External Interrupt Configuration, x=1 to 4 EXTICR1: for pin0~pin3 , EXTICR2: for pin4~pin7, etc |
| EXTI_ | EXTI_IMR | Interrupt Mask |
|  | EXTI_FTSR EXTI_RTSR | Falling/Rising Trigger Selection |

Schematic

B. Register Setting

(Digital Input Setting)

- Enable GPIO peripheral clock                 	RCC->AHB1ENR

- Configure DigitalIn pin

(EXTI Setting)

- Enable SYSCFG peripheral clock. 		RCC->APB2ENR

- Connect the corresponding external line to GPIO 	SYSCFG->EXTICR

- Configure the trigger edge. 			EXTI->FTSR/RTSR

- Configure Interrupt mask 			EXTI->IMR

- Enable EXTI. 					EXTI->IMR

(NVIC Setting)

- Configure the priority of EXTI interrupt request. 	NVIC_SetPriority()

- Enable EXTI interrupt request. 			NVIC_EnableIRQ()

(EXTI Use)

- Create user codes in handler			EXTIx_IRQHandler()

- Clear pending bit after interrupt call

III. Tutorial

A. Register Configuration

1. Pin Initialization & Set LED and Push-button

LED: Port A Pin 5 / Output / Push-Pull / No Pull-Up & No Pull-Down

Push-button: Port C Pin 13 / Input / No Pull-Up & No Pull-Down

2. Enable Peripheral Clock: SYSCFGEN

- RCC_APB2ENR: Enable SYSCFG

RCC->APB2ENR |= 1UL << 14

// Paste RCC_APB2ENR register map

3. EXTI Initialization & Connect Push-button to EXTI line

- SYSCFG_EXTICR4: Connect PC_13(push-button) to EXTI13 line

SYSCFG->EXTICR[3] &=~ 15UL ≪ 4   		 // clear bits [3:0]

SYSCFG->EXTICR[3] |= 2UL << 4 		// set to 0010 for PC[13]

// past register map here

- EXTI_FTSR: Enable Falling Trigger

EXTI->FTSR |= 1 << 13   // TR13=1

// past register map here

- EXTI_IMR: Interrupt NOT masked (Enable)

EXTI->IMR |= 1 << 13	// MR13 = 1

// past register map here

B. Programming

Procedure

- Create a new folder ‘EC/Tutorial/TU_EXTI/’

- Open the program ‘Keil uVision5’ and create a new project.

- Name the project as ‘TU_EXTI’.

- Create a new item called ‘TU_EXTI.c’ and use the given source code Click here to download

- This is an example code for turning LED on/off with the button input trigger.

- Fill in the empty spaces in the code.

- Run the program and check your result.

- Your tutorial report must be submitted to LMS

Appendix

See here for MCU resources

- Pin Configuration of NUCLE-F401RE

- LED/Button Circuit Diagram